Methodology used in picking and locating water wells

Water well locations were first plotted using the Oregon Department of Water Resources GRID (Groundwater Resource Information) database; which plotted all wells at section , quareter section or quarter-quarter section centroids. A plot of all wells deeper than 100 ft was next superimposed on georeferenced raster images of USGS 7 1/2 topographic maps (USGS DRG's). Well locations were adjusted to match more detailed locations described on actual water well logs. Wells with street addresses were located within appropriate city blocks atop an overlay of MapInfo StreetInfo files (proprietary enhanced TIGER files with address coding). Wells with bearing and distance data were plotted from raster image section corners.

Important deep wells were field located by direct contact with the well owners or -in some cases- direct visual confirmation. Locations of important new wells were plotted on appropriate topographic maps during field visits as they were being drilled.